# ENGINEERING CHANGE REQUEST (ECR)

Warehouse Robotics Division

# 1. ECR IDENTIFICATION

| ECR Number: | ECR-2025-0017 | Date Submitted: | 2025-12-13 |
| --- | --- | --- | --- |
| Originator: | Gaurav Shetty | Department: | Warehouse Robotics - Reliability |
| Email: | gaurav.shetty276@gmail.com | Phone: | +1 (555) 014-2291 |

# 2. PRIORITY & ROBOT INFORMATION

| Priority Level | Robot Model(s) Affected |
| --- | --- |
| ☐ CRITICAL - Safety issue, robot inoperable ☒ HIGH - Performance degradation, customer impact ☐ MEDIUM - Improvement, cost reduction ☐ LOW - Documentation, cosmetic | Model: WR-450 Autonomous Mobile Robot (AMR) Serial Number Range: SN 0450-2100 to SN 0450-2899 All Models: ☐ |

# 3. PROBLEM STATEMENT / OPPORTUNITY

Issue Description

Clearly describe the problem or opportunity: During peak pallet moves, the AMR intermittently triggers an emergency slow-down due to false-positive obstacle detections on the front LiDAR. This causes stop-and-go behavior, queueing in narrow aisles, and occasional mission aborts. Log review suggests the issue correlates with reflective shrink-wrap + low-angle scans; the filter thresholds in the current firmware appear too sensitive.

Source / Origin

☒ Field Failure / Customer Report ☒ Internal Testing / QA ☐ Design Improvement ☐ Cost Reduction Opportunity ☐ Regulatory / Compliance Requirement ☐ Supplier-Initiated Change ☐ Safety Concern Other:

Failure Mode / Frequency (if applicable)

How often does this occur? Intermittent (~1 in 200 missions; higher in reflective aisles) Severity: ☐ Critical ☒ High ☐ Medium ☐ Low Customer Impact: ☒ Yes (deployed units affected) ☐ No

# 4. PROPOSED CHANGE

Affected Subsystems (Check all that apply)

☐ Mechanical / Structural ☐ Electrical / Power Systems ☒ Sensors (LiDAR, Cameras, Ultrasonic, etc.) ☐ Control Systems / Electronics ☒ Software / Firmware ☒ Safety Systems (E-Stop, Collision Avoidance, Speed Limiting) ☐ Communication / Network ☐ Battery / Charging System ☒ Navigation / Localization Other:

Detailed Description of Proposed Change

1) Firmware update: adjust LiDAR obstacle-cluster filtering to reduce false positives on high-reflectivity returns while preserving safety margins. 2) Sensor mount shim: add a 1.5mm spacer to slightly adjust the LiDAR pitch angle to reduce floor/reflective-wrap artifacts in the near field. 3) Calibration update: re-run factory calibration routine post-shim for affected serials.

Affected Components / BOM Items

Part Number(s): LDR-22-008 (Front LiDAR Assembly), SHIM-01-001 (New), FAST-SS-M4x12 (existing) Drawing / Specification Changes: Update DRW-LDR-22-008 Rev C → Rev D to include shim + torque note. Software Version Changes: nav-fw v3.8.2 → v3.8.3 (LiDAR filter threshold + reflective return handling)

Justification / Business Case

Why is this change necessary? Reduces stop-and-go events and mission aborts in customer sites with reflective packaging, improving throughput and reducing support tickets. Change is low risk and can be rolled out via OTA for firmware plus a simple field shim where required.

# 5. IMPACT ANALYSIS

SAFETY IMPACT (CRITICAL)

Does this change affect robot safety functions? ☒ YES ☐ NO If YES, describe safety impact: The change modifies perception filtering used by collision avoidance. Safety validation and regression testing are required to ensure detection performance remains within spec. Safety systems affected (E-Stop, Collision Avoidance, Speed Limiting, Personnel Detection): Collision avoidance / speed limiting; perception pipeline thresholds

Performance Impact

Speed / Throughput: +6% estimated aisle throughput (reduced unnecessary slow-downs) Payload Capacity: No change Battery Life / Runtime: +1–2% (less stop/start acceleration) Navigation Accuracy: No change expected; verify in localization regression Reliability / MTBF: Expected improvement (reduced mission aborts); target +10% MTBF for perception faults

Interchangeability

Can old and new parts be mixed? ☒ YES ☐ NO Forward compatible (new part works in old robots)? ☒ YES ☐ NO Backward compatible (old part works with new design)? ☒ YES ☐ NO

Field Retrofit / Deployed Units

Field retrofit required for deployed robots? ☒ YES ☐ NO Number of units in field affected: Approx. 320 deployed units in reflective-wrap customer environments Software update required? ☒ YES ☐ NO OTA capable? ☒ YES ☐ NO Hardware replacement required? ☐ YES ☒ NO Customer notification required? ☒ YES ☐ NO

Supply Chain / Supplier Impact

New suppliers required? ☐ YES ☒ NO Supplier qualification needed? ☐ YES ☒ NO Lead time impact: Shim can be sourced internally; 2 weeks for first batch MOQ (Minimum Order Quantity) considerations: MOQ 500 units if externalized; otherwise printed/in-house cut

# 6. COST ANALYSIS

| Cost Category | Estimated Amount ($) |
| --- | --- |
| Engineering Development Costs | $18,000 (firmware + validation) |
| Tooling / Fixturing Changes | $0 (no new fixtures required) |
| Unit Cost Impact ($ per robot) | +$0.35/unit (shim + packaging) |
| Inventory Obsolescence | $0 (no hard replacement of existing LiDAR assemblies) |
| Field Retrofit Costs (if applicable) | $12,000 (labor + travel estimate for affected sites where shim is required) |
| Testing / Validation Costs | $7,500 (lab + pilot run) |
| Certification / Compliance Costs | $0 (no new certifications expected; safety doc update only) |
| Total Estimated Cost | $37,850 |
| Expected Savings / Benefits (if applicable) | Reduced support tickets + downtime; est. $60k/year support savings across top customers |

# 7. TESTING  VALIDATION REQUIREMENTS

Required Testing (Check all that apply)

☒ Functional Testing ☒ Safety Validation (Critical for safety-related changes) ☐ Environmental Testing (Temperature, Humidity, Dust) ☒ Endurance / Reliability Testing ☐ EMC / EMI Testing ☒ Software Regression Testing ☒ Integration Testing ☒ Navigation / Localization Testing ☒ Payload / Stress Testing ☐ Battery / Power Testing ☐ Customer Acceptance Testing Other:

Test Plan Reference

Test Plan Number / Document: TP-WR450-PERCEP-019 (LiDAR reflective returns) + REG-NAV-042 Estimated Testing Duration: 2.5 weeks (lab) + 1 week pilot

# 8. IMPLEMENTATION PLAN

Proposed Effectivity

Proposed Effectivity Date: 2026-01-15 Effectivity Point: ☒ Serial Number: SN 0450-2100 and later (firmware OTA); shim retrofit for flagged sites only ☐ Production Date: SN 0450-2100 and later (firmware OTA); shim retrofit for flagged sites only Lead Time Required: 3–4 weeks (firmware release + pilot + shim kitting)

Documentation Updates Required

☐ User Manual ☒ Maintenance Guide ☐ Installation Instructions ☒ Service Bulletins ☒ Safety Documentation ☒ Training Materials ☐ Spare Parts Catalog Other:

# 9. ATTACHMENTS

Please attach the following supporting documentation (check all that apply):

☒ Photos or videos of the issue

☒ Failure analysis reports

☐ Proposed drawings or CAD files

☒ Test data or results

☐ Risk assessment (FMEA for safety-related changes)

☒ Customer feedback or complaints

☐ Supplier documentation

☒ Software code or version control references

Other:

# 10. APPROVAL WORKFLOW

| Role | Name / Signature | Date |
| --- | --- | --- |
| Engineering Manager | Alicia Tran | 2025-12-13 |
| Quality Manager | Marcus Patel | 2025-12-14 |
| Safety Engineer (Mandatory for safety changes) | Elena Rodríguez | 2025-12-14 |
| Software Lead (If software affected) | Noah Kim | 2025-12-13 |
| Manufacturing / Operations | Priya Nair | 2025-12-16 |
| Supply Chain | Daniel Brooks | 2025-12-16 |
| Customer Service / Field Support | Samantha Lee | 2025-12-17 |
| Final Approval: Engineering Director / Change Board | VP Engineering (TBD) |  |

--- END OF ECR FORM ---

For internal use: Upon approval, this ECR will be converted to an ECO (Engineering Change Order)